TEACHER PAGES

# Relicensing Ferrand Dam: Sediment, Fish, and Cold Water

Reading passage · Grade 12 · English Language Arts

A grade 12 informational passage on a fictional dam's license renewal, with downstream oxygen and temperature data students read against the text.

| Time | 40 minutes |
| --- | --- |
| Prep | No prep |
| Materials | None |
| Answer key | At the end of this document |
| License | Public domain (CC0). Copy, edit, and share freely, without attribution. |

## Objectives

- Students will explain how a stratified reservoir changes the temperature and oxygen of the water released below a dam.

- Students will resolve an apparent contradiction between a stated principle and a data table.

- Students will describe two trade-offs a license renewal imposes on a dam owner.

## Standards

RST.11-12.7 (Common Core ELA): Integrate and evaluate multiple sources of information presented in diverse formats and media (e.g., quantitative data, video, multimedia) in order to address a question or solve a problem.

How this material addresses it: Questions 3 and 4 require students to read the two-station oxygen and temperature table against the passage's account of reservoir layering, resolving an apparent contradiction that neither source settles alone.

## Vocabulary

Dissolved oxygen: Dissolved oxygen is the oxygen gas mixed into water that fish and other underwater animals breathe. Cold, moving water holds more oxygen than warm, still water.

Also: relicensing, stratification, sediment load, minimum flow, reaeration

## Teacher overview

A hydropower dam is licensed for a term measured in decades, and the renewal of that license is one of the few moments when everything about how the dam operates is open to change. This passage follows a fictional renewal and treats the reservoir as a physical system with consequences downstream, including water that leaves the dam cold and short of oxygen. The paired table sets up a genuine puzzle: the passage states that cold water can hold more oxygen than warm water, and the data below the dam appear to contradict it. Question 4 is the heart of the assignment, and students should not be given the resolution in advance.

## Differentiation

### Support

Provide a cross-section sketch of the reservoir showing the warm surface layer, the cold deep layer, and the low-level outlet, with an arrow for the release. Give students the table with the August row highlighted, and allow questions 1, 2, and 3 to be answered in phrases rather than full sentences.

### Extension

Ask students to write the owner's one-paragraph response to a proposed minimum flow condition, using only what the passage supplies and following the agencies' instruction to show the arithmetic rather than assert it. They must state which figures the owner would need to supply and why the record cannot settle the question without them.

### ELL supports

Pre-teach "license," "renewal," "layer," "release," and "spawning" with a short definition and one everyday example for each. Provide a two-column chart labeled "what the dam does for people" and "what the dam does to the river" for students to fill in while reading, and offer a sentence frame for question 4: "Cold water can hold more oxygen, but the deep water ___, so ___."

Name	   Date

# Relicensing Ferrand Dam: Sediment, Fish, and Cold Water

Grade 12 · English Language Arts

## Passage

Ferrand Dam has stood across the Windemere River since 1926. The dam and the river are fictional, but the situation is common. Ferrand is small by the standards of its industry. Its powerhouse is rated at eleven megawatts, which is a fraction of a percent of what a project such as Hoover Dam is rated to produce, since the Bureau of Reclamation puts that plant's capacity at about 2,080 megawatts. Hoover's actual generation is lower today, because years of drought and low water in Lake Mead have cut it by about 30 percent. Ferrand's owner sells the power to a regional cooperative, and the reservoir behind the dam supports a marina, two campgrounds, and a town beach.

Dams like Ferrand operate under a federal license, and the license eventually expires. Renewal is not a formality. It reopens the operating conditions themselves, including how much water the dam must release, when it must release it, what it must do for migrating fish, and what it must monitor. Agencies, tribes, county governments, anglers, and the owner all file comments, and the record they build is the basis for the new license. For a dam approaching its hundredth year, renewal is the moment when a century of accumulated effects gets priced.

Three of those effects dominate the Ferrand record. The first is temperature. A reservoir separates into layers in summer. Sun warms the surface, the warm layer floats, and the deep water stays cold and still. Ferrand releases through a low-level outlet, so the water leaving the dam in August comes from the bottom of the reservoir. It arrives downstream far colder than the river would be without the dam. Cold water suits trout and suits the anglers who fish for them, and it suits native fish adapted to a warmer summer river much less well.

The second effect is oxygen. Water can dissolve only so much oxygen, and cold water can hold more of it than warm water can. That capacity is a ceiling on what the water can hold rather than a description of what it does hold. Deep reservoir water sits under the warm layer for months with no contact with the air, while bacteria on the bottom consume oxygen as they break down settled organic material. By late summer the deep layer can be well below the amount actually available at the surface. Most fish need dissolved oxygen above 5 mg/L, a benchmark agencies apply throughout the country. Water released from a low-level outlet in August can arrive under that benchmark even though it is the coldest water in the system.

The third effect is sediment. A river carries sand and gravel, and a reservoir stops that movement almost completely. The material settles into a growing delta where the river enters the reservoir, which slowly takes away storage the dam was built to provide. Below the dam the river runs clear and hungry. It picks up what it can from the bed, coarsens the channel, and receives nothing from upstream to replace the gravel that spawning fish require. A century of that arithmetic has left the reach below Ferrand paved with cobble.

The dam also changes when the river's water arrives. Snowmelt that would have run past in May is held and released through the late summer, which steadies flow for the powerhouse and for the marina. Storage of that kind is why dams are built. It comes with a loss that a free-flowing reach does not have, since a wide reservoir surface evaporates water all summer that the river would otherwise have carried downstream.

The conditions under discussion in the renewal follow from these three effects. A multi-level intake would let operators draw from a warmer layer when the river below needs warmth. Turbine venting would pull air into the water as it passes the runner and raise the oxygen of the release. A minimum flow requirement would set a floor under how low the river can drop when power is worth the most. Periodic sediment sluicing would move some gravel past the dam. Each condition costs the owner generation, capital, or both, and the owner has told the record that some combinations would make the project uneconomic to operate. The agencies have asked the owner to show that arithmetic rather than assert it.

The table gives four months of monitoring from two stations, one directly below the dam and one twelve miles downstream at Saltash Bridge.

| Month | Oxygen below dam, mg/L | Temperature below dam, degrees Celsius | Oxygen at Saltash Bridge, mg/L | Temperature at Saltash Bridge, degrees Celsius |
| --- | --- | --- | --- | --- |
| June | 8.1 | 9 | 9.2 | 14 |
| July | 6.2 | 8 | 8.4 | 17 |
| August | 4.3 | 8 | 7.6 | 19 |
| September | 5.8 | 9 | 8.1 | 17 |

## Questions

- According to the passage, what does renewal of the license reopen? Name three specific things.

- Where in the reservoir does Ferrand's outlet draw its water, and what two properties does that water have by late summer?

- Using the table, name the month in which oxygen below the dam fell under the benchmark the passage gives for most fish, and give both the measured value and the benchmark.

- The passage states that cold water can hold more oxygen than warm water. The table shows the colder water below the dam carrying less oxygen than the warmer water at Saltash Bridge in every month. Explain why both can be true.

- The passage says the river below the dam runs "clear and hungry." Explain what that phrase means in terms of sediment, and name two consequences the passage gives.

- Explain how the dam changes the timing of the river's water, and name the loss the passage says a reservoir adds that a free-flowing reach does not have.

- Choose one condition under discussion in the renewal. Explain what problem it addresses and what it costs the owner, using only what the passage states.

ANSWER KEY · FOR TEACHERS

# Relicensing Ferrand Dam: Sediment, Fish, and Cold Water

Reading passage · Grade 12 · English Language Arts

- Answer: Renewal reopens the dam's operating conditions. Accept any three of how much water the dam must release, when it must release it, what it must do for migrating fish, and what it must monitor.

- Answer: Ferrand releases through a low-level outlet, so the water comes from the bottom of the reservoir. By late summer that deep water is both very cold and low in dissolved oxygen.

- Answer: August, when oxygen below the dam measured 4.3 mg/L against a benchmark of 5 mg/L for most fish. The table gives the measurement and the passage gives the benchmark.

- Answer: Temperature sets a ceiling on how much oxygen water can hold, not how much it actually holds. The deep layer sits sealed under the warm layer for months while bacteria consume oxygen breaking down settled material, so it arrives well below its ceiling. Farther downstream the water warms, which lowers the ceiling, but it also tumbles and mixes with air, which brings the actual oxygen back up toward that lower ceiling.

- Answer: The reservoir traps the sand and gravel the river used to carry, so the water leaving the dam has no sediment load and picks up material from the bed instead. Accept any two consequences the passage names: the delta in the reservoir takes away storage, the channel below coarsens, and the gravel that spawning fish require is never replaced.

- Answer: The dam holds snowmelt that would have passed in May and releases it through late summer, steadying flow for the powerhouse and the marina. The added loss is evaporation from the wide reservoir surface all summer, water the river would otherwise have carried downstream.

- Answer: Accept any one condition with an accurate problem and cost. Because the passage says only that each condition costs "generation, capital, or both," accept any of these costs when the student ties it to the condition; the pairings below are examples. A multi-level intake addresses release temperature and costs capital. Turbine venting addresses low oxygen in the release and costs capital. A minimum flow requirement addresses low river levels and costs generation at the hours when power is worth the most. Sediment sluicing addresses the gravel shortage below the dam and costs both.