TEACHER PAGES

# Sitwell Basin Aquifer Storage and Recovery Worksheet

Worksheet · Grade 12 · Engineering & Design

A grade 12 worksheet on a fictional aquifer storage and recovery project: porosity from core samples, recoverable storage in acre-feet and gallons, and injection math.

| Time | 40 minutes |
| --- | --- |
| Prep | No prep |
| Materials | Calculator |
| Answer key | At the end of this document |
| License | Public domain (CC0). Copy, edit, and share freely, without attribution. |

## Objectives

- Students will calculate porosity from core-sample void and total volumes.

- Students will estimate an aquifer zone's recoverable storage from area, thickness, and porosity.

- Students will convert seasonal injection volumes between gallons and acre-feet and compute recovery time.

## Standards

HSN-Q.A.1 (Common Core Math): Use units as a way to understand problems and to guide the solution of multi-step problems; choose and interpret units consistently in formulas; choose and interpret the scale and the origin in graphs and data displays.

How this material addresses it: Items 5 through 8 carry acres, feet, acre-feet, gallons, and MGD through a single storage calculation, the multi-step unit chain this standard requires.

HSN-Q.A.2 (Common Core Math): Define appropriate quantities for the purpose of descriptive modeling.

How this material addresses it: Items 4 and 6 separate bulk volume from recoverable storage and make porosity the per-unit quantity that decides which zone models the district's available supply, which is the quantity-selection work this standard describes.

## Vocabulary

Porosity: Porosity measures how much empty space a rock or soil has between its grains, which tells you how much water it can hold. A sponge has high porosity, while a piece of glass has almost none.

Also: aquifer storage and recovery, core sample, recharge well, recoverable storage

## Teacher overview

Aquifer storage and recovery, or ASR, treats an aquifer as an underground reservoir: a utility injects treated water through wells in wet years, lets the aquifer hold it, and pumps it back during drought. Sizing such a project depends on porosity, because only the void space between the grains holds water. This worksheet walks students through the fictional Sitwell Basin ASR project from two core samples to a full seasonal water budget, using the standard porosity formula and the acre-foot conversions that groundwater engineers use daily.

## Differentiation

### Support

Provide the porosity formula worked through with a third practice core (for instance, 400 cubic centimeters total and 80 void, giving 20 percent) before students attempt items 2 and 3, and confirm item 5 as a checkpoint since items 6, 7, and 9 all build on it.

### Extension

Have students compute how many 120-day injection seasons the fictional district would need to fill Zone A's recoverable storage completely at 1.5 MGD, assuming no losses, and then explain why a real project would stop well short of 100 percent. (1,472 ÷ 552.4 is about 2.7 seasons.)

### ELL supports

Introduce "porosity" with a jar-of-marbles visual, water filling only the gaps, and label a cross-section showing the recharge well, the storage zone, and the direction of injection and recovery. Allow items 1, 4, and 11 to be answered with a labeled sketch plus a single sentence.

Name	   Date

# Sitwell Basin Aquifer Storage and Recovery Worksheet

Grade 12 · Engineering & Design

## Student Worksheet

The fictional Sitwell Basin water district is designing an aquifer storage and recovery project. Use these facts throughout. Porosity (%) = void volume ÷ total volume × 100. One acre-foot equals 325,851 gallons. The district's recharge well injects 1.5 MGD, and its recovery pumping rate is 2.0 MGD.

- In one or two sentences, explain what an ASR project does with treated water in wet years and in dry years.

- A core sample from candidate Zone A has a total volume of 400 cubic centimeters, with 92 cubic centimeters of void space between grains. Calculate Zone A's porosity as a percent.

- A core from deeper Zone B has a total volume of 400 cubic centimeters and 58 cubic centimeters of void space. Calculate Zone B's porosity as a percent, to the nearest tenth.

- Which zone can store more water per unit of volume, and why? Answer using your two porosity values.

- The district selects Zone A, which spreads under 160 acres with an average thickness of 40 feet. Compute the zone's bulk volume in acre-feet.

- For this worksheet, use a simplified upper-bound model that sets recoverable storage equal to bulk volume × porosity. Real projects use specific yield or storativity, which give smaller values. Using Zone A's porosity from item 2, compute the recoverable storage in acre-feet.

- Convert the recoverable storage from item 6 into gallons.

- In one wet season, the recharge well injects 1.5 MGD for 120 days. Convert the total injected volume into acre-feet, to the nearest tenth.

- What percent of Zone A's recoverable storage does that one season of injection fill? Round to the nearest tenth of a percent.

- A drought arrives and the district pumps the stored water back at 2.0 MGD. Assuming it recovers exactly the volume injected in item 8, how many days does recovery take?

- Give one physical reason the district might recover somewhat less water than it injected.

ANSWER KEY · FOR TEACHERS

# Sitwell Basin Aquifer Storage and Recovery Worksheet

Worksheet · Grade 12 · Engineering & Design

- Answer: It stores water underground in wet years and withdraws it in dry years. In wet years the district injects treated water through wells into the aquifer; in drought it pumps the stored water back out for supply.

- Answer: 23 percent. 92 ÷ 400 × 100 = 23 percent porosity.

- Answer: 14.5 percent. 58 ÷ 400 × 100 = 14.5 percent porosity.

- Answer: Zone A. Its porosity (23 percent versus 14.5 percent) means more of each cubic foot is open void space available to hold water.

- Answer: 6,400 acre-feet. 160 acres × 40 feet = 6,400 acre-feet of bulk volume.

- Answer: 1,472 acre-feet. 6,400 acre-feet × 0.23 = 1,472 acre-feet of recoverable storage.

- Answer: 479,652,672 gallons. 1,472 acre-feet × 325,851 gallons per acre-foot = 479,652,672 gallons.

- Answer: 552.4 acre-feet. 1.5 MGD × 120 days = 180 million gallons, and 180,000,000 ÷ 325,851 = 552.4 acre-feet.

- Answer: 37.5 percent. 552.4 ÷ 1,472 = 0.375, or 37.5 percent of the recoverable storage.

- Answer: 90 days. 180,000,000 gallons ÷ 2,000,000 gallons per day = 90 days of recovery pumping.

- Answer: Some stored water migrates or mixes and cannot all be captured. Acceptable answers include water drifting beyond the well's capture zone with natural groundwater flow, mixing with poorer-quality native water, or being held too tightly in small pores to yield.