TEACHER PAGES

# Sable Point Water System: Treatment and Hydraulics Test

Quiz & test questions · Grade 9 · Engineering & Design

A grade 9 engineering unit test worth 30 points, mixing multiple-choice, short-answer, and computation items on head, pressure, flow, detention, filters, and removal.

| Time | 30 minutes |
| --- | --- |
| Prep | No prep |
| Materials | Calculator |
| Answer key | At the end of this document |
| License | Public domain (CC0). Copy, edit, and share freely, without attribution. |

## Objectives

- Students will apply head, pressure, and flow conversions to distribution and treatment problems.

- Students will calculate detention time, filter loading rate, and removal efficiency and interpret the results.

## Standards

HSN-Q.A.1 (Common Core Math): Use units as a way to understand problems and to guide the solution of multi-step problems; choose and interpret units consistently in formulas; choose and interpret the scale and the origin in graphs and data displays.

How this material addresses it: Computation items 3 through 7 require students to carry units through pressure, flow-conversion, detention-time, filter-loading, and removal-efficiency work, choosing consistent units at each step.

HSN-Q.A.3 (Common Core Math): Choose a level of accuracy appropriate to limitations on measurement when reporting quantities.

How this material addresses it: Items 5 and 6 ask students to report detention time and a loading rate at a level of accuracy that matches the plant's design ranges and supports a clear judgment.

## Vocabulary

Detention time: Detention time is how long water stays inside a tank or basin before flowing out. Water must stay long enough for each treatment step, like settling or disinfecting, to finish its job.

Also: head, pressure, loading rate, removal efficiency

## Teacher overview

This 30-minute unit test (30 points total) checks whether students can apply the core engineering math of the water unit. It uses one fictional system, Sable Point, and mixes two multiple-choice items, five computation items, and one short-answer item. The items cover head and pressure, flow conversions, detention time, filter loading, and removal efficiency.

## Differentiation

### Support

Provide a conversion reference card with the conversion factors used in items 3 and 4, and set up the first computation item (150 x 0.433) so students complete only the final step. Allow a calculator for every computation item.

### Extension

Ask students to find the pump run time to refill the 150,000-gallon basin at 1,000 gpm, and to explain how detention time and loading rate would both change if the plant's flow rose by 20 percent.

### ELL supports

Provide a labeled diagram of a tank, a hill, and a home for item 8, and give a sentence frame such as "The hilltop home needs a booster pump because the tank sits ___ the home, so there is ___ head." Allow numeric answers to be shown as labeled steps.

Name	   Date

# Sable Point Water System: Treatment and Hydraulics Test

Grade 9 · Engineering & Design

## Assessment

The Sable Point regional water system (a fictional system used for this test) serves a small city. Use 1 foot = 0.433 psi and 1 MGD = 694 gpm where needed.

- (2 points) One psi of pressure equals how many feet of water head?

A) 0.433 feet

B) 2.31 feet

C) 7.48 feet

D) 8.34 feet

- (2 points) A distribution system usually operates in which pressure range?

A) 10 to 15 psi

B) 40 to 80 psi

C) 100 to 150 psi

D) 200 to 300 psi

- (4 points) A tank's water surface sits 150 feet above a home. Find the water pressure at the home, using 0.433 psi per foot. Round to the nearest whole psi.

- (4 points) A plant's design flow is 2.5 MGD. Convert this flow to gallons per minute (gpm).

- (4 points) A sedimentation basin holds 150,000 gallons and the plant flow is 1.0 MGD. Find the detention time in hours. (Detention time in days = volume divided by daily flow; multiply by 24 for hours.)

- (4 points) A filter has 200 square feet of surface area and receives 1,000 gpm. Find the filter loading rate in gpm per square foot.

- (5 points) A plant lowers its influent BOD from 200 mg/L to an effluent of 20 mg/L. Find the removal efficiency as a percent. (Removal efficiency = (influent minus effluent) divided by influent, times 100.)

- (5 points) Explain why a neighborhood on a hill above the water tank often needs a booster pump to keep good pressure. Use the idea of head in your answer.

ANSWER KEY · FOR TEACHERS

# Sable Point Water System: Treatment and Hydraulics Test

Quiz & test questions · Grade 9 · Engineering & Design

- Answer: B) 2.31 feet.
One psi equals 2.31 feet of head, the standard conversion between pressure and the height of a water column.

- Answer: B) 40 to 80 psi.
Distribution systems typically run between 40 and 80 psi, with most states requiring at least 20 psi at every connection.

- Answer: 65 psi.
Pressure = 150 feet x 0.433 psi per foot = 64.95, which rounds to 65 psi.

- Answer: 1,735 gpm.
2.5 MGD x 694 gpm per MGD = 1,735 gpm.

- Answer: 3.6 hours.
Detention in days = 150,000 gallons / 1,000,000 gallons per day = 0.15 day. In hours: 0.15 x 24 = 3.6 hours.

- Answer: 5.0 gpm/sq ft.
Loading rate = 1,000 gpm / 200 square feet = 5.0 gpm per square foot, inside the usual 2 to 6 range.

- Answer: 90%.
Removal efficiency = (200 - 20) / 200 x 100 = 180 / 200 x 100 = 90 percent.

- Answer: The hill adds head the pump must overcome.
Homes on the hill sit above the tank's water surface, so gravity alone cannot give them enough pressure. A booster pump adds the extra head, about 0.433 psi for every foot of rise, needed to keep pressure in the normal range at the tap.