TEACHER PAGES

# Highgrove Plant Alum Dose: Feed Rate and Chemical Cost

Lesson plan · Grade 10 · Math

A grade 10 math lesson using the pounds formula to find a fictional plant's alum feed rate in pounds per day, then its daily and yearly chemical cost.

| Time | 55 minutes |
| --- | --- |
| Prep | Low prep |
| Materials | • Calculator • Printed handout • Two clear jars of water, one settled clear and one still cloudy • Index cards |
| Answer key | At the end of this document |
| License | Public domain (CC0). Copy, edit, and share freely, without attribution. |

## Objectives

- Students will calculate a coagulant feed rate in pounds per day using the pounds formula.

- Students will find the daily and yearly chemical cost and the savings from a lower dose.

## Standards

HSN-Q.A.1 (Common Core Math): Use units as a way to understand problems and to guide the solution of multi-step problems; choose and interpret units consistently in formulas; choose and interpret the scale and the origin in graphs and data displays.

How this material addresses it: Handout items 1, 4, and 5 carry mg/L, MGD, and the 8.34 constant through the pounds formula, the consistent multi-step unit analysis this standard requires.

HSA-CED.A.1 (Common Core Math): Create equations and inequalities in one variable and use them to solve problems. Include equations arising from linear and quadratic functions, and simple rational and exponential functions.

How this material addresses it: Handout items 6 and 7 use one-variable equations relating dose change to pounds saved and dollars saved, the problem-solving this standard describes.

## Vocabulary

Alum: Alum is the most common chemical that water plants add to make tiny dirt particles clump together. A pinch of alum can help a whole jar of muddy water turn clear.

Also: coagulant, dose, pounds formula

## Teacher overview

Drinking-water plants add a coagulant such as alum to make small particles clump so they settle out. An operator runs a jar test to find the lowest dose that still works, because every extra milligram per liter is chemical the plant pays for. This lesson uses a fictional plant at Highgrove and the pounds formula, feed (lbs/day) = dose (mg/L) x flow (MGD) x 8.34, to price a dose. A common misconception this lesson addresses is that a higher dose is always safer; overdosing wastes money, lowers pH, and makes more sludge to handle.

## Procedure

Hook (5 min): Show two jars that started with the same cloudy water, one that has settled clear and one still cloudy, and ask what an operator adds to make particles settle. Introduce alum and the idea that dose costs money.

Direct instruction (12 min): Write the pounds formula on the board: feed (lbs/day) = dose (mg/L) x flow (MGD) x 8.34. Explain that 8.34 is the weight of a gallon of water in pounds, and that daily cost is feed times the price per pound.

Guided modeling (13 min): Model with a fictional plant treating 3.0 MGD at a 30 mg/L alum dose. Together, find the feed of 30 x 3.0 x 8.34 = 750.6 pounds per day, labeling each unit. This worked example is for instruction only; students do not turn it in.

Independent practice (20 min): Students complete the Student Handout for the Highgrove plant. Circulate and check item 1's feed rate before students continue, since items 2 and 3 build on it.

Closure and formative check (5 min): On an index card, students write one sentence explaining why a lower dose that still clears the water saves money. Collect and scan for the idea that unused chemical is wasted cost.

## Differentiation

### Support

Provide the pounds formula and item 1 already set up as 25 x 6.0 x 8.34, so students focus on the arithmetic and the later comparisons. Allow calculators and a units checklist.

### Extension

Have students find the yearly savings from the 20 mg/L dose at the plant's 6.0 MGD flow, then decide whether that saving would justify a fictional $2,000 upgrade to the jar-testing equipment, and explain their reasoning.

### ELL supports

Pre-teach "coagulant," "dose," and "feed rate" with a picture of particles clumping and settling after alum is added. Provide a sentence frame for item 8: "The lowest effective dose matters because ___."

Name	   Date

# Highgrove Plant Alum Dose: Feed Rate and Chemical Cost

Grade 10 · Math

## Student Handout

The Highgrove Water Treatment Plant is a fictional plant, and its numbers are set for this handout. Use feed (lbs/day) = dose (mg/L) x flow (MGD) x 8.34, and show your work.

Plant data:

- Flow treated: 6.0 MGD

- Alum dose from the jar test: 25 mg/L

- Alum price: $0.30 per pound

- Find the alum feed rate in pounds per day at the 25 mg/L dose.

- Find the daily alum cost at $0.30 per pound.

- Find the yearly alum cost. Use 365 days.

- Flow rises to 8.0 MGD at the same 25 mg/L dose. Find the new feed rate in pounds per day.

- A second jar test finds 20 mg/L still works. At 6.0 MGD, find the feed rate at 20 mg/L.

- Find the pounds of alum saved per day by dropping from 25 mg/L to 20 mg/L at 6.0 MGD.

- Find the daily cost savings from that lower dose, at $0.30 per pound.

- Explain why finding the lowest effective dose matters, beyond just saving chemical money.

ANSWER KEY · FOR TEACHERS

# Highgrove Plant Alum Dose: Feed Rate and Chemical Cost

Lesson plan · Grade 10 · Math

- Answer: 1,251 lbs/day.
25 mg/L x 6.0 MGD x 8.34 = 150 x 8.34 = 1,251 pounds per day.

- Answer: $375.30.
1,251 pounds per day x $0.30 per pound = $375.30 per day.

- Answer: $136,985.
$375.30 per day x 365 days = $136,984.50, which rounds to about $136,985 per year.

- Answer: 1,668 lbs/day.
25 mg/L x 8.0 MGD x 8.34 = 200 x 8.34 = 1,668 pounds per day.

- Answer: 1,001 lbs/day.
20 mg/L x 6.0 MGD x 8.34 = 120 x 8.34 = 1,000.8, which rounds to 1,001 pounds per day.

- Answer: 250 lbs/day.
1,251 - 1,000.8 = 250.2, or about 250 pounds saved per day. This also equals 5 mg/L x 6.0 x 8.34 = 250.2.

- Answer: about $75 per day.
250.2 pounds x $0.30 per pound = $75.06, or about $75 saved per day.

- Answer: Overdosing lowers pH and makes more sludge, so the lowest effective dose protects quality too.
Extra alum beyond what the water needs drops the pH and increases the sludge the plant must remove and dispose of, so the right dose saves money and keeps the treated water in range.