TEACHER PAGES

# Silverton Standpipe Geometry and Hydraulics Quiz

Quiz & test questions · Grade 10 · Math

A 30-point grade 10 quiz mixing multiple choice, computation, and short answer on cylinder volume, pressure from height, and continuity for a fictional water district.

| Time | 30 minutes |
| --- | --- |
| Prep | No prep |
| Materials | Calculator |
| Answer key | At the end of this document |
| License | Public domain (CC0). Copy, edit, and share freely, without attribution. |

## Objectives

- Students will compute the volume of a cylindrical standpipe in cubic feet and gallons.

- Students will apply pressure-from-height and the continuity equation to a distribution system.

## Standards

HSG-MG.A.1 (Common Core Math): Use geometric shapes, their measures, and their properties to describe objects (e.g., modeling a tree trunk or a human torso as a cylinder).

How this material addresses it: Items 1 through 3 model a standpipe as a cylinder and compute its volume in cubic feet and gallons, the object-as-geometric-shape modeling this standard describes.

HSN-Q.A.1 (Common Core Math): Use units as a way to understand problems and to guide the solution of multi-step problems; choose and interpret units consistently in formulas; choose and interpret the scale and the origin in graphs and data displays.

How this material addresses it: Items 5 and 6 carry units through the continuity equation and a cfs-to-gpm conversion, the multi-step unit reasoning this standard names.

## Vocabulary

Standpipe: A standpipe is a tall, cylindrical water storage tank that sits directly on the ground instead of standing on legs like a water tower. It stores water and helps maintain steady pressure in the pipes nearby.

Also: cylinder volume, pressure, continuity

## Teacher overview

This 30-point quiz checks whether grade 10 students can apply geometry and hydraulics to a distribution system at the fictional Silverton Water District. It mixes two multiple-choice items, four computation items, and two short-answer items across cylinder volume, pressure from water height, and the continuity equation. Each item states its point value.

## Differentiation

### Support

Provide a formula card listing the four relationships used on the quiz, and mark item 2 with the values already placed (0.785 × 30² × 40) so students can focus on the arithmetic. Allow a calculator on every item.

### Extension

Ask students to find the water height, in feet, that would produce exactly 65 psi at a home in the fictional district, working backward through pressure = height × 0.433. Then have them state whether that height is above or below the standpipe's 40-foot water depth and what that implies for hilltop service.

### ELL supports

Provide a labeled diagram of the standpipe showing diameter, water depth, and the height of water above two homes at different elevations. Allow short-answer items 7 and 8 to be completed with the numeric work plus a brief phrase rather than a full sentence.

Name	   Date

# Silverton Standpipe Geometry and Hydraulics Quiz

Grade 10 · Math

## Assessment

The Silverton Water District (a fictional district) stores water in a cylindrical standpipe 30 feet in diameter with 40 feet of water depth. Use these facts where needed: cylinder volume (cu ft) = 0.785 × diameter² × depth; 1 cubic foot = 7.48 gallons; pressure (psi) = height (ft) × 0.433; and flow Q = area × velocity. Most states require at least 20 psi at every connection, and utilities aim for 40 to 80 psi.

- (3 points) Which expression gives the standpipe's volume in cubic feet?

A) 0.785 × diameter × depth

B) 0.785 × diameter² × depth

C) 2.31 × diameter² × depth

D) diameter² × depth

- (4 points) Using volume = 0.785 × diameter² × depth, find the standpipe's volume in cubic feet.

- (4 points) Convert the volume from item 2 to gallons using 1 cubic foot = 7.48 gallons.

- (3 points) The water surface stands 120 feet above a home. Using pressure = height × 0.433, the pressure at the home is closest to:

A) 28 psi

B) 52 psi

C) 92 psi

D) 277 psi

- (4 points) A distribution pipe has a cross-sectional area of 0.8 square feet and carries water at 3.5 feet per second. Using Q = area × velocity, find the flow in cubic feet per second.

- (4 points) Convert the flow from item 5 to gallons per minute using 1 cfs = 448.8 gpm.

- (3 points) The home's pressure from item 4 is about 52 psi. Utilities aim for 40 to 80 psi. Is 52 psi within that target range? Answer yes or no and explain in one sentence.

- (5 points) A second home sits 200 feet below the standpipe's water surface. Find its water pressure, then explain in one sentence why the district might install a pressure-reducing valve there.

ANSWER KEY · FOR TEACHERS

# Silverton Standpipe Geometry and Hydraulics Quiz

Quiz & test questions · Grade 10 · Math

- Answer: B) 0.785 × diameter² × depth. This is the standard formula for the volume of a cylinder written with diameter, so it is the correct choice.

- Answer: 28,260 cu ft. Volume = 0.785 × 30² × 40 = 0.785 × 900 × 40 = 28,260 cubic feet.

- Answer: 211,385 gal. 28,260 cubic feet × 7.48 gallons per cubic foot = 211,384.8, about 211,385 gallons.

- Answer: B) 52 psi. Pressure = 120 × 0.433 = 51.96, which rounds to 52 psi, the closest choice.

- Answer: 2.8 cfs. Q = area × velocity = 0.8 square feet × 3.5 feet per second = 2.8 cubic feet per second.

- Answer: 1,257 gpm. 2.8 cfs × 448.8 gpm per cfs = 1,256.64, about 1,257 gpm.

- Answer: Yes. 52 psi falls between 40 and 80 psi, so the home's pressure is inside the target range utilities aim for.

- Answer: 86.6 psi. Pressure = 200 × 0.433 = 86.6 psi. Because that is above the 80 psi comfort ceiling, the district might install a pressure-reducing valve to protect the home's plumbing and fixtures.