TEACHER PAGES

# Verdant Hollow Recovery Facility: Nutrient Load and Removal

Worksheet · Grade 10 · Math

A grade 10 worksheet using the pounds formula to find nitrogen and phosphorus loads, removal efficiency, and permit compliance at a fictional treatment plant.

| Time | 40 minutes |
| --- | --- |
| Prep | No prep |
| Materials | Calculator |
| Answer key | At the end of this document |
| License | Public domain (CC0). Copy, edit, and share freely, without attribution. |

## Objectives

- Students will calculate nitrogen and phosphorus mass loads in pounds per day using the pounds formula.

- Students will find removal efficiency and compare treated effluent against a permit limit.

## Standards

HSN-Q.A.1 (Common Core Math): Use units as a way to understand problems and to guide the solution of multi-step problems; choose and interpret units consistently in formulas; choose and interpret the scale and the origin in graphs and data displays.

How this material addresses it: Items 1, 2, 4, and 5 carry mg/L, MGD, and the 8.34 constant through the pounds formula, the consistent multi-step unit reasoning this standard targets.

HSN-Q.A.2 (Common Core Math): Define appropriate quantities for the purpose of descriptive modeling.

How this material addresses it: Item 8 asks students to decide whether a load in pounds per day or a concentration in mg/L is the better quantity for comparing plants of different sizes.

## Vocabulary

Effluent: Effluent is the cleaned water that leaves a treatment plant at the end of the process. It is tested carefully before being released to a river, lake, or the ocean.

Also: nutrient load, pounds formula, removal efficiency

## Teacher overview

Nitrogen and phosphorus are nutrients that plants need, yet in a river they feed algae and lower dissolved oxygen. A treatment plant reports how much of each nutrient it removes as a mass, in pounds per day, not just as a concentration. This worksheet uses the fictional town of Verdant Hollow and the pounds formula, load (lbs/day) = concentration (mg/L) x flow (MGD) x 8.34. Teaching note: before students start, work one load out loud so they see that flow, not concentration alone, sets how many pounds reach the river each day.

## Differentiation

### Support

Give students a printed formula strip with load (lbs/day) = concentration x flow x 8.34 and the removal-efficiency formula already written out. Work item 1 together, labeling each number with its unit, before students continue on their own.

### Extension

Have students find the pounds of nitrogen the river would receive in a full 30-day month at the current effluent load, and explain how a wetter month with higher flow would change that monthly total even if the concentration held steady.

### ELL supports

Pre-teach "nutrient," "load," "influent," and "effluent" with a simple labeled diagram showing water entering and leaving a plant. Provide a sentence frame for item 8: "Pounds per day is a better comparison because ___."

Name	   Date

# Verdant Hollow Recovery Facility: Nutrient Load and Removal

Grade 10 · Math

## Student Worksheet

The town of Verdant Hollow runs a water resource recovery facility. This plant is fictional, and its numbers are set for this worksheet. Use the pounds formula for every load: load (lbs/day) = concentration (mg/L) x flow (MGD) x 8.34.

Plant data:

- Flow treated: 3.2 MGD

- Total nitrogen: 35 mg/L coming in, 8 mg/L going out

- Total phosphorus: 6 mg/L coming in, 1 mg/L going out

- Permit limit for total nitrogen leaving the plant: 10 mg/L

- Find the nitrogen load coming into the plant, in pounds per day.

- Find the nitrogen load leaving the plant, in pounds per day.

- Find the nitrogen removal efficiency as a percent. Use removal efficiency (%) = (in - out) / in x 100.

- Find the phosphorus load coming into the plant, in pounds per day.

- Find the pounds of phosphorus the plant removes each day.

- The plant's nitrogen leaves at 8 mg/L. Does that meet the 10 mg/L permit limit? By how many mg/L does it pass or miss?

- Find the phosphorus removal efficiency as a percent.

- A smaller plant treats less flow but reports a higher nitrogen concentration than Verdant Hollow. Explain why comparing pounds per day, rather than mg/L alone, tells you which plant sends more nitrogen to the river.

ANSWER KEY · FOR TEACHERS

# Verdant Hollow Recovery Facility: Nutrient Load and Removal

Worksheet · Grade 10 · Math

- Answer: 934 lbs/day.
35 mg/L x 3.2 MGD x 8.34 = 112 x 8.34 = 934.08, which rounds to 934 pounds per day.

- Answer: 214 lbs/day.
8 mg/L x 3.2 MGD x 8.34 = 25.6 x 8.34 = 213.5, which rounds to 214 pounds per day.

- Answer: 77 percent.
(35 - 8) / 35 x 100 = 27 / 35 x 100 = 77.1, or about 77 percent. Students who use the loads get the same result: (934 - 214) / 934 x 100 is about 77 percent.

- Answer: 160 lbs/day.
6 mg/L x 3.2 MGD x 8.34 = 19.2 x 8.34 = 160.1, which rounds to 160 pounds per day.

- Answer: 133 lbs/day.
Phosphorus removed is (6 - 1) = 5 mg/L. Then 5 x 3.2 x 8.34 = 16 x 8.34 = 133.4, which rounds to 133 pounds per day.

- Answer: Yes, by 2 mg/L.
The nitrogen leaves at 8 mg/L, which is below the 10 mg/L limit, so it passes with a margin of 10 - 8 = 2 mg/L.

- Answer: 83 percent.
(6 - 1) / 6 x 100 = 5 / 6 x 100 = 83.3, or about 83 percent.

- Answer: Pounds per day account for flow, so they show the true amount reaching the river.
A high concentration at a low flow can still be fewer pounds than a lower concentration at a high flow. The river responds to total mass, so pounds per day is the fair comparison between plants of different sizes.