TEACHER PAGES

# Halloway Standpipe: Storage, Fire Flow, and Pressure

Math problem set · Grade 12 · Math

A grade 12 problem set sizing a fictional town's standpipe, converting elevation to service pressure, and checking whether stored water meets a two-hour fire-flow demand.

| Time | 45 minutes |
| --- | --- |
| Prep | No prep |
| Materials | Calculator |
| Answer key | At the end of this document |
| License | Public domain (CC0). Copy, edit, and share freely, without attribution. |

## Objectives

- Students will compute the storage volume of a cylindrical standpipe in gallons.

- Students will convert elevation head to service pressure and back using the standard constants.

- Students will test stored water against a fire-flow demand stated in gpm and duration.

## Standards

HSG-MG.A.1 (Common Core Math): Use geometric shapes, their measures, and their properties to describe objects (e.g., modeling a tree trunk or a human torso as a cylinder).

How this material addresses it: Problem 1 models the Halloway standpipe as a cylinder and uses its diameter and depth to find storage volume, the shape-to-object modeling this standard names.

HSN-Q.A.1 (Common Core Math): Use units as a way to understand problems and to guide the solution of multi-step problems; choose and interpret units consistently in formulas; choose and interpret the scale and the origin in graphs and data displays.

How this material addresses it: Problems 2 through 7 carry units through elevation-to-pressure conversions and a fire-flow volume and duration, keeping feet, psi, gallons, and gpm consistent.

## Vocabulary

Standpipe: A standpipe is a tall, cylindrical water storage tank that sits directly on the ground instead of standing on legs like a water tower. It stores water and helps maintain steady pressure in the pipes nearby.

Total dynamic head (TDH): Total dynamic head is the total amount of lift a pump has to provide to move water where it needs to go, adding up the height it must lift the water, the friction it must overcome in the pipes, and the pressure needed at the far end. Engineers calculate it to pick a pump that is powerful enough for the job.

Also: elevation head, service pressure, fire flow

## Teacher overview

The fictional town of Halloway stores its water in a tall cylindrical standpipe and relies on elevation, not a running pump, to hold pressure at the tap. This set has students size the standpipe, translate its water-surface elevation into service pressure, and test whether the stored volume can sustain a two-hour fire flow. A common misconception is that a wider tank produces more pressure; students should see that pressure at a service connection depends only on the height of water above that point, so the tank's diameter changes the stored volume but not the pressure.

## Differentiation

### Support

Provide a printed constant card with the three formulas, and pre-compute the tank radius and squared diameter for problem 1 so students can focus on the volume and conversion steps.

### Extension

Ask students to find the tank diameter that would double the storage volume in problem 1 while keeping the 60-foot depth, and confirm that this larger tank leaves the Elm Street pressure in problem 2 unchanged.

### ELL supports

Provide a labeled diagram of the standpipe showing diameter, water depth, and the 130-foot height above the service connection as three separate measurements. Allow students to answer problem 8 with a labeled sketch and a short caption.

## Extensions

Have students find the height and diameter of a real standpipe or elevated tank in a nearby town, estimate its storage and the pressure it delivers to a ground-level home, and compare those figures with the Halloway values.

Name	   Date

# Halloway Standpipe: Storage, Fire Flow, and Pressure

Grade 12 · Math

## Problems

The town of Halloway operates one cylindrical standpipe. The tank is 50 feet in diameter with a water depth of 60 feet, and its water surface sits 130 feet above the Elm Street service connection. Use these constants: cylinder volume (cu ft) = 0.785 x diameter squared x depth, one cubic foot holds 7.48 gallons, and one foot of water height equals 0.433 psi.

- Find the volume of water in the standpipe in cubic feet, then convert it to gallons.

- Find the water pressure at the Elm Street connection, given the water surface is 130 feet above it. Round to the nearest hundredth of a psi.

- A fire at a warehouse needs a flow of 1,500 gpm sustained for 2 hours. How many gallons does that fire flow require?

- What percent of the full standpipe volume from problem 1 does the fire-flow demand in problem 3 use? Round to the nearest tenth.

- Ignoring any inflow, how long could the full standpipe alone sustain the 1,500 gpm fire flow? Give the answer in minutes and in hours.

- A pump refilling the tank must overcome a static lift of 130 feet plus 18 feet of friction loss. Find the total dynamic head in feet, then express it in psi.

- During the fire the water surface drops 12 feet. Find the new pressure at Elm Street, and state whether it stays above the 20 psi minimum that state rules require at every connection.

- Explain why the standpipe's diameter changes the answer to problem 1 but not the answer to problem 2.

ANSWER KEY · FOR TEACHERS

# Halloway Standpipe: Storage, Fire Flow, and Pressure

Math problem set · Grade 12 · Math

- Answer: 880,770 gallons.
Volume = 0.785 x 50 squared x 60 = 0.785 x 2,500 x 60 = 117,750 cubic feet; 117,750 x 7.48 = 880,770 gallons.

- Answer: 56.29 psi.
130 feet x 0.433 psi per foot = 56.29 psi at the Elm Street connection.

- Answer: 180,000 gallons.
1,500 gpm x 2 hours x 60 minutes per hour = 1,500 x 120 = 180,000 gallons.

- Answer: 20.4%.
180,000 / 880,770 x 100 = 20.44%, about 20.4 percent of the full tank.

- Answer: 587.2 minutes, or about 9.8 hours.
880,770 gallons / 1,500 gpm = 587.18 minutes; 587.18 / 60 = 9.79 hours.

- Answer: 148 feet, or 64.08 psi.
Total dynamic head = 130 + 18 = 148 feet; 148 x 0.433 = 64.08 psi.

- Answer: 51.09 psi, which stays above 20 psi.
The water surface falls to 130 - 12 = 118 feet above Elm Street; 118 x 0.433 = 51.09 psi, well above the 20 psi minimum.

- Answer: Diameter sets volume, but only height sets pressure.
The volume formula multiplies by the diameter squared, so a wider tank holds much more water. Pressure at a connection depends only on the vertical height of water above it, which the diameter does not change, so problem 2 is unaffected by the tank's width.