TEACHER PAGES

# Glenmara Treatment Plant: A Process Math Quiz

Quiz & test questions · Grade 9 · Math

A grade 9 unit quiz on the fictional Glenmara plant, with computation and multiple-choice items on detention time, filter loading, removal efficiency, and dosing.

| Time | 40 minutes |
| --- | --- |
| Prep | No prep |
| Materials | Calculator |
| Answer key | At the end of this document |
| License | Public domain (CC0). Copy, edit, and share freely, without attribution. |

## Objectives

- Students will calculate detention time, loading rate, removal efficiency, and chlorine feed.

- Students will convert a plant flow between MGD and gallons per minute.

- Students will identify treatment steps and a finished-water turbidity limit.

## Standards

HSN-Q.A.1 (Common Core Math): Use units as a way to understand problems and to guide the solution of multi-step problems; choose and interpret units consistently in formulas; choose and interpret the scale and the origin in graphs and data displays.

How this material addresses it: Items 3, 8, and 9 carry gallons, MGD, mg/L, and the 8.34 constant through detention time, dosing, and flow conversion so each result keeps consistent units.

HSG-MG.A.2 (Common Core Math): Apply concepts of density based on area and volume in modeling situations (e.g., persons per square mile, BTUs per cubic foot).

How this material addresses it: Item 5 treats filter loading rate as a density of gallons per minute per square foot, the flow-per-area modeling this standard targets.

## Vocabulary

Sedimentation: Sedimentation is the step where water sits almost still in a big basin so the heavy clumps of dirt can sink to the bottom. The cleaner water on top then moves on to the filters.

Detention time: Detention time is how long water stays inside a tank or basin before flowing out. Water must stay long enough for each treatment step, like settling or disinfecting, to finish its job.

Turbidity: Turbidity is a measure of how cloudy or murky water looks because of tiny particles floating in it. Clear water has low turbidity, and muddy water has high turbidity.

Also: loading rate, removal efficiency, pounds formula

## Teacher overview

This 40-minute unit quiz, worth 30 points total, checks grade 9 skills on the fictional Glenmara treatment plant. It mixes multiple-choice, short-answer, and computation items. Students move through the treatment train: sedimentation detention time, filter loading rate, removal efficiency, and chlorine dosing, then convert a plant flow and recall a finished-water turbidity limit.

## Differentiation

### Support

Provide a formula strip listing detention time, loading rate, removal efficiency, and the pounds formula, each with one worked example that is not on the quiz. Allow a calculator on all computation items and read the multiple-choice stems aloud.

### Extension

Ask students to find the largest flow, in gpm, the item-5 filter at the fictional plant can take while staying at or below 6 gpm per square foot, and then to convert that flow to MGD.

### ELL supports

Provide the formula strip with a small labeled picture for each step: settling tank, filter bed, and a chlorine feed pump. Let students answer item 6 with a sentence frame: "The loading rate is ___ the range because 5 is between ___ and ___."

Name	   Date

# Glenmara Treatment Plant: A Process Math Quiz

Grade 9 · Math

## Assessment

Glenmara (a fictional plant) treats river water. Use these facts: detention time = volume divided by flow; loading rate = flow in gpm divided by area in square feet; removal efficiency, in percent, = (influent minus effluent) divided by influent, times 100; pounds per day = mg/L, times MGD, times 8.34; one cubic foot holds 7.48 gallons; one MGD equals 694 gpm.

- (2 points) Which step lets particles settle out of slow-moving water?

A) Coagulation

B) Sedimentation

C) Disinfection

D) Distribution

- (2 points) Which expression gives chemical feed in pounds per day?

A) mg/L, times MGD, times 8.34

B) mg/L divided by MGD

C) gpm, times 2.31

D) NTU, times 0.433

- (4 points) A sedimentation basin holds 80,000 gallons and passes 480,000 gallons per day. Find the detention time in hours.

- (2 points) A typical sedimentation basin is designed for how many hours of detention?

A) 2 to 4 hours

B) 20 to 40 hours

C) 1 to 2 minutes

D) 10 to 12 hours

- (4 points) A filter passes 900 gpm across 180 square feet of surface. Find the loading rate in gpm per square foot.

- (3 points) Conventional filters run 2 to 6 gpm per square foot. Is the item-5 loading rate inside that range? Explain in one sentence.

- (4 points) The influent turbidity is 40 NTU and the settled water is 10 NTU. Find the removal efficiency as a percent.

- (4 points) The plant doses chlorine at 2.5 mg/L into a 2 MGD flow. Find the chlorine feed in pounds per day.

- (3 points) Convert the plant's 2 MGD flow to gallons per minute.

- (2 points) Finished drinking water must stay at or below what turbidity in 95 percent of samples?

A) 0.3 NTU

B) 5 NTU

C) 15 NTU

D) 40 NTU

ANSWER KEY · FOR TEACHERS

# Glenmara Treatment Plant: A Process Math Quiz

Quiz & test questions · Grade 9 · Math

- Answer: B) Sedimentation. In sedimentation, slow-moving water lets heavy particles fall to the bottom before filtration.

- Answer: A) mg/L, times MGD, times 8.34. This is the pounds formula for daily chemical feed.

- Answer: 4 hours. Detention time = 80,000 divided by 480,000 = 0.1667 day; 0.1667 times 24 = 4 hours.

- Answer: A) 2 to 4 hours. Sedimentation basins are commonly designed for 2 to 4 hours of detention.

- Answer: 5 gpm/sq ft. Loading rate = 900 gpm divided by 180 square feet = 5 gpm per square foot.

- Answer: Yes. 5 gpm per square foot falls between 2 and 6 gpm per square foot, so it is inside the range.

- Answer: 75 %. (40 minus 10) divided by 40, times 100 = 30 divided by 40, times 100 = 75 percent.

- Answer: 41.7 lb/day. 2.5 mg/L times 2 MGD times 8.34 = 41.7 pounds per day.

- Answer: 1,388 gpm. 2 MGD times 694 gpm per MGD = 1,388 gpm.

- Answer: A) 0.3 NTU. Finished water must stay at or below 0.3 NTU in 95 percent of samples.