TEACHER PAGES

# Halcyon Ridge Stormwater: A Rational-Method Runoff Lesson

Lesson plan · Grade 9 · Math

A 50-minute grade 9 lesson where students use the rational method Q = C i A to compare runoff from pavement and lawn at Halcyon Ridge and size a rain garden.

| Time | 50 minutes |
| --- | --- |
| Prep | Low prep |
| Materials | • Printed handout • Calculators • Photos of a paved lot and a grassy field |
| Answer key | At the end of this document |
| License | Public domain (CC0). Copy, edit, and share freely, without attribution. |

## Objectives

- Students will use the rational method Q = C x i x A to find peak stormwater runoff.

- Students will compare runoff from surfaces with different runoff coefficients.

- Students will size a rain garden to hold a roof's storm runoff.

## Standards

HSN-Q.A.1 (Common Core Math): Use units as a way to understand problems and to guide the solution of multi-step problems; choose and interpret units consistently in formulas; choose and interpret the scale and the origin in graphs and data displays.

How this material addresses it: Students carry units through the rational method Q = C x i x A and the roof-runoff and cubic-foot conversions in handout questions 1 through 6, keeping units consistent.

## Vocabulary

Rain garden: A rain garden is a shallow planted dip in the ground that catches rain running off a roof or driveway and lets it soak slowly into the soil. Its deep-rooted plants help clean the water.

Also: rational method, runoff coefficient, peak runoff, impervious

## Teacher overview

On the fictional Halcyon Ridge campus, a paved lot sheds far more storm runoff than a lawn of the same size. Engineers estimate peak runoff with the rational method, Q = C x i x A, where the runoff coefficient C is about 0.9 for pavement and 0.2 for lawn. In this lesson students use the rational method to compare surfaces, then size a rain garden to hold a roof's runoff from a 1-inch storm. A common misconception is thinking area alone sets the runoff, when the surface type, through C, matters just as much.

## Procedure

Launch (7 min): Show two photos, a paved lot and a grassy field, and ask which sheds more water in a storm on the fictional Halcyon Ridge campus. Record predictions on the board.

Model the rational method (13 min): Introduce Q = C x i x A, where C is the runoff coefficient (about 0.9 for pavement, 0.2 for lawn), i is rainfall intensity in inches per hour, and A is area in acres. Work one example: a 0.5-acre parking lot with C = 0.9 and i = 2 inches per hour gives Q = 0.9 x 2 x 0.5 = 0.9 cfs. Note that in these units the answer comes out in cubic feet per second.

Guided practice (10 min): Repeat the calculation treating the same lot as lawn (C = 0.2), then compare the two results and draw out how the runoff coefficient drives the difference.

Independent practice (15 min): Students complete handout questions 1 through 7, including the roof-runoff and rain-garden sizing steps.

Closure / exit ticket (5 min): Students answer handout question 8 on a slip, finding the peak runoff from a smaller pavement area. Scan for correct use of Q = C x i x A before dismissal.

## Differentiation

### Support

Provide the rational-method formula and the two runoff coefficients on a card, plus one fully worked example that is not on the handout. Give the conversion for question 4 in words, and allow a calculator throughout.

### Extension

Ask students to find how much the peak runoff drops if half the parking lot is converted to lawn, and to explain the result using the two runoff coefficients.

### ELL supports

Pre-teach "runoff," "coefficient," and "rain garden" with a picture of rain hitting pavement and rushing off, next to rain soaking into a garden. Provide a sentence frame for question 7.

Name	   Date

# Halcyon Ridge Stormwater: A Rational-Method Runoff Lesson

Grade 9 · Math

## Student Handout

All figures below describe the fictional Halcyon Ridge campus.

- A parking lot has runoff coefficient C = 0.9, rain intensity i = 2 inches per hour, and area A = 0.5 acre. Use Q = C x i x A to find the peak runoff in cubic feet per second (cfs).

- A lawn of the same size has C = 0.2, with the same i = 2 inches per hour and A = 0.5 acre. Find its peak runoff.

- How many times more peak runoff comes from the pavement than from the lawn?

- Convert the parking lot's 0.9 cfs to gallons per minute, using 1 cfs equal to 448.8 gpm.

- The school roof is 8,000 square feet. A 1-inch storm makes runoff of area x inches x 0.623 gallons. Find the roof runoff in gallons.

- A rain garden must hold that 4,984 gallons. Using 1 cubic foot equal to 7.48 gallons, find the volume needed in cubic feet.

- Explain why replacing pavement with lawn or a rain garden lowers the peak runoff Q.

- A 0.25-acre pavement area has C = 0.9 and i = 2 inches per hour. Find its peak runoff Q.

ANSWER KEY · FOR TEACHERS

# Halcyon Ridge Stormwater: A Rational-Method Runoff Lesson

Lesson plan · Grade 9 · Math

- Answer: 0.9 cfs. Q = 0.9 x 2 x 0.5 = 0.9 cubic feet per second.

- Answer: 0.2 cfs. Q = 0.2 x 2 x 0.5 = 0.2 cubic feet per second.

- Answer: 4.5 times. 0.9 cfs / 0.2 cfs = 4.5, so the pavement sheds 4.5 times the peak runoff.

- Answer: 404 gpm. 0.9 cfs x 448.8 gpm per cfs = 403.9, about 404 gallons per minute.

- Answer: 4,984 gallons. 8,000 sq ft x 1 inch x 0.623 = 4,984 gallons of roof runoff.

- Answer: 666 cu ft. 4,984 gallons / 7.48 gallons per cubic foot = 666.3, about 666 cubic feet.

- Answer: A lawn or rain garden has a lower runoff coefficient C, so with the same rain and area the product C x i x A is smaller, which means less peak runoff.

- Answer: 0.45 cfs. Q = 0.9 x 2 x 0.25 = 0.45 cubic feet per second.