TEACHER PAGES

# Dellacre Farm Irrigation Demand: Net, Gross, and Efficiency

Math problem set · Grade 12 · Science

A grade 12 problem set on irrigation water math: students convert crop needs to acre-feet, apply system efficiency to find gross demand, and size the delivery flow.

| Time | 45 minutes |
| --- | --- |
| Prep | No prep |
| Materials | Calculator |
| Answer key | At the end of this document |
| License | Public domain (CC0). Copy, edit, and share freely, without attribution. |

## Objectives

- Students will convert a crop's seasonal water depth over a field area into a net volume in acre-feet and gallons.

- Students will use application efficiency to convert net crop need into gross water that must be diverted.

- Students will size an average delivery flow in MGD and gpm and compare irrigation methods.

## Standards

HSN-Q.A.1 (Common Core Math): Use units as a way to understand problems and to guide the solution of multi-step problems; choose and interpret units consistently in formulas; choose and interpret the scale and the origin in graphs and data displays.

How this material addresses it: Students carry units through the inches-to-acre-feet, acre-feet-to-gallons, and gallons-to-flow chain in problems 1 through 8, the multi-step dimensional analysis this standard requires.

HSN-Q.A.2 (Common Core Math): Define appropriate quantities for the purpose of descriptive modeling.

How this material addresses it: Problems 4 and 9 have students distinguish net crop need from gross applied water and choose the right quantity for a diversion request, the modeling decision this standard names.

HS-ESS3-4 (NGSS): Evaluate or refine a technological solution that reduces impacts of human activities on natural systems.

How this material addresses it: Problems 9 and 10 evaluate drip irrigation as a technological solution that cuts the gross water Dellacre Farm must divert, quantifying the roughly 67 acre-feet saved over the sprinkler system.

## Vocabulary

Acre-foot: The amount of water it takes to cover one acre of land, about the size of a football field, one foot deep. Western states use acre-feet to measure large amounts of water.

Drip irrigation: Watering plants through tubes that release water slowly, drop by drop, right at the roots. Very little water is wasted along the way.

Also: net irrigation requirement, application efficiency, gross demand

## Teacher overview

The fictional Dellacre Farm irrigates a 150-acre field from a canal, and this problem set works through how much water the farm actually has to divert to meet its crop. Students turn a seasonal crop need into a volume, then account for the water a sprinkler system loses so they can find the larger gross amount that must be delivered. A common error is multiplying the net crop need by the efficiency instead of dividing by it. Because a system loses water, the gross applied amount is always larger than the net need, so the efficiency goes in the denominator.

## Differentiation

### Support

Provide problem 4 with the division set up (300 / 0.75 = ___) and a printed reminder that gross is larger than net. Work problems 1 and 2 together as a class so the depth-times-area step is clear before students continue.

### Extension

Ask students to find the annual water savings in gallons if the drip system in problem 9 is used every season for five years, and to weigh that savings against a reason a farm might still hesitate to install drip irrigation.

### ELL supports

Pre-teach "net," "gross," and "efficiency" with a picture of water leaving a sprinkler where some reaches the crop and some is lost. Provide a sentence frame for problem 10 and allow a labeled diagram in place of a paragraph.

## Extensions

Have students look up the main crop grown in their region and its typical seasonal water requirement, then repeat problems 1 through 4 with that figure to estimate the gross water a local field of the same size would need.

Name	   Date

# Dellacre Farm Irrigation Demand: Net, Gross, and Efficiency

Grade 12 · Science

## Problems

Use these values and conversions for the fictional Dellacre Farm:

- Field area: 150 acres

- Net crop water requirement for the season: 24 inches

- Sprinkler system application efficiency: 75 percent

- Irrigation season length: 100 days

- Conversions: 1 acre-foot = 325,851 gallons; depth in feet = inches / 12; 1 MGD = 694 gpm.

- Convert the 24-inch seasonal crop requirement to a depth in feet.

- Calculate the net water volume the crop needs over the 150-acre field, in acre-feet.

- Convert the net requirement from problem 2 to gallons.

- Calculate the gross water that must be applied, accounting for the 75 percent application efficiency (gross = net / efficiency), in acre-feet.

- Calculate the extra water lost to inefficiency, in acre-feet, as the difference between the gross and net volumes.

- Convert the gross applied water from problem 4 to gallons.

- The gross water is delivered evenly over the 100-day season. Calculate the average delivery flow in MGD. Round to the nearest hundredth.

- Express the average delivery flow from problem 7, before rounding, in gallons per minute, using 1 MGD = 694 gpm. Round to the nearest whole gpm.

- The farm switches to drip irrigation at 90 percent application efficiency for the same 300 acre-feet of net need. Calculate the new gross demand in acre-feet and the water saved compared with the sprinkler system in problem 4.

- Explain why raising application efficiency lowers the water a farm must divert from the canal, and name one method that raises efficiency.

ANSWER KEY · FOR TEACHERS

# Dellacre Farm Irrigation Demand: Net, Gross, and Efficiency

Math problem set · Grade 12 · Science

- Answer: 2.0 feet.
24 inches / 12 inches per foot = 2.0 feet.

- Answer: 300 acre-feet.
150 acres x 2.0 feet = 300 acre-feet of net crop need.

- Answer: 97,755,300 gallons.
300 acre-feet x 325,851 gallons per acre-foot = 97,755,300 gallons, about 97.8 million gallons.

- Answer: 400 acre-feet.
Gross = net / efficiency = 300 acre-feet / 0.75 = 400 acre-feet.

- Answer: 100 acre-feet.
400 acre-feet gross minus 300 acre-feet net = 100 acre-feet lost to inefficiency.

- Answer: 130,340,400 gallons.
400 acre-feet x 325,851 gallons per acre-foot = 130,340,400 gallons, about 130.3 million gallons.

- Answer: 1.30 MGD.
130,340,400 gallons / 100 days = 1,303,404 gallons per day, which is 1.30 MGD.

- Answer: 905 gpm.
1.3034 MGD x 694 gpm per MGD = 904.6, which rounds to 905 gpm.

- Answer: 333.33 acre-feet gross, saving about 66.67 acre-feet.
Gross for drip = 300 acre-feet / 0.90 = 333.33 acre-feet. Compared with the 400 acre-feet the sprinkler system needed, the drip system saves 400 - 333.33 = 66.67 acre-feet, about 67 acre-feet.

- Answer: Higher efficiency means less water is lost, so less must be diverted.
When more of the applied water reaches the crop's root zone, a smaller gross amount meets the same net need, so the farm diverts less from the canal. Drip irrigation, better scheduling with soil-moisture sensors, or lining canals to cut seepage all raise efficiency.