TEACHER PAGES

# Harlow Heights Roof Runoff: A Rain-Capture Problem Set

Math problem set · Grade 7 · Science

A grade 7 science problem set on a fictional school's rooftop rain capture, finding runoff gallons, cistern percent captured, and how surface type changes runoff.

| Time | 45 minutes |
| --- | --- |
| Prep | No prep |
| Materials | Calculator |
| Answer key | At the end of this document |
| License | Public domain (CC0). Copy, edit, and share freely, without attribution. |

## Objectives

- Students will calculate rooftop runoff in gallons using roof area, rainfall depth, and the 0.623 conversion.

- Students will find the percent of runoff a cistern captures and the percent runoff change from swapping surface types.

## Standards

7.RP.A.2 (Common Core Math): Recognize and represent proportional relationships between quantities, including identifying the constant of proportionality and representing proportional relationships by equations.

How this material addresses it: Problems 1, 4, and 8 treat roof runoff as proportional to rainfall depth and roof area with the constant 0.623, identifying that constant of proportionality as this standard requires.

7.RP.A.3 (Common Core Math): Use proportional relationships to solve multistep ratio and percent problems. Examples: simple interest, tax, markups and markdowns, gratuities and commissions, fees, percent increase and decrease, percent error.

How this material addresses it: Problems 2, 3, and 7 are multistep percent problems finding the percent of runoff a cistern captures and the percent reduction from replacing pavement with lawn, the reasoning this standard develops.

MS-ESS3-3 (NGSS): Apply scientific principles to design a method for monitoring and minimizing a human impact on the environment.

How this material addresses it: Problems 2, 3, and 5 size the cistern that captures rooftop runoff, and problems 6 and 7 quantify how swapping pavement for lawn cuts runoff from the same area, the stormwater and land-use impacts this standard's clarification names as impacts to monitor and minimize.

## Vocabulary

Cistern: A large tank that stores rainwater collected from a roof so it can be used later, usually for watering plants.

Also: runoff, runoff coefficient, impermeable

## Teacher overview

When rain falls on a roof, most of it runs off instead of soaking in, and that runoff can be captured for reuse or sent to a storm drain. The amount of runoff depends on the roof area, the depth of rain, and a conversion of 0.623 gallons for each square foot of roof per inch of rain. This set follows a fictional school, Harlow Heights Elementary, as it captures rooftop rain in a cistern. A common misconception this set corrects is thinking rainfall in inches is already a volume; students must multiply area by rainfall depth and by 0.623 to get gallons, not read the inches as gallons.

## Differentiation

### Support

Provide the runoff formula on a reference card and complete problem 1 together, writing each multiplication step. Allow students to reuse the 7,476 gallons from problem 1 in problems 2 and 3 without recomputing it.

### Extension

Ask students to find what cistern size would be needed to capture all of the runoff from the 1.5-inch storm in problem 1, and then to estimate how many such storms per year the school could fully capture given its yearly total from problem 8.

### ELL supports

Pre-teach "runoff," "cistern," and "coefficient" with a labeled picture of rain hitting a roof and flowing into a tank. Provide a worked unit reminder card showing that square feet times inches times 0.623 gives gallons.

Name	   Date

# Harlow Heights Roof Runoff: A Rain-Capture Problem Set

Grade 7 · Science

## Problems

Harlow Heights Elementary is a fictional school. Its roof is 8,000 square feet. Use the conversion: gallons of runoff equals roof area in square feet times rainfall in inches times 0.623. This gives all the rain that falls on the roof; a real gutter and cistern system collects somewhat less, but for these problems assume all of it flows to the cistern.

- A storm drops 1.5 inches of rain on the roof. How many gallons of runoff does the roof shed?

- The school's cistern holds 2,500 gallons. What percent of the runoff from problem 1 does a full cistern capture? Round to the nearest tenth of a percent.

- How many gallons of runoff overflow past the 2,500-gallon cistern in problem 1?

- A smaller storm drops only 0.5 inch of rain. How many gallons of runoff does the roof shed then?

- Does the runoff from problem 4 fit inside the empty 2,500-gallon cistern? If so, how many gallons of space are left?

- Runoff coefficients describe how much water runs off a surface: pavement is about 0.9 and lawn is about 0.2. For the same area and rainfall, how many times more runoff comes from pavement than from lawn?

- The school replaces some pavement with lawn. Because runoff is proportional to the coefficient, the runoff from that area drops from a coefficient of 0.9 to 0.2. What is the percent decrease in runoff for that area? Round to the nearest tenth.

- Over a year the roof receives 40 inches of rain in total. How many gallons of runoff does the 8,000-square-foot roof shed in a year?

ANSWER KEY · FOR TEACHERS

# Harlow Heights Roof Runoff: A Rain-Capture Problem Set

Math problem set · Grade 7 · Science

- Answer: 7,476 gallons.
8,000 times 1.5 times 0.623 = 12,000 times 0.623 = 7,476 gallons.

- Answer: 33.4%.
2,500 divided by 7,476 times 100 = 33.44 percent, which rounds to 33.4 percent.

- Answer: 4,976 gallons.
7,476 − 2,500 = 4,976 gallons overflow past the cistern.

- Answer: 2,492 gallons.
8,000 times 0.5 times 0.623 = 4,000 times 0.623 = 2,492 gallons.

- Answer: Yes, 8 gallons of space are left.
2,492 is less than 2,500, so it fits, and 2,500 − 2,492 = 8 gallons of space remain.

- Answer: 4.5 times.
0.9 divided by 0.2 = 4.5, so pavement sheds 4.5 times as much runoff as lawn for the same area and rain.

- Answer: 77.8%.
(0.9 − 0.2) divided by 0.9 times 100 = 0.7 divided by 0.9 times 100 = 77.78 percent, which rounds to 77.8 percent.

- Answer: 199,360 gallons.
8,000 times 40 times 0.623 = 320,000 times 0.623 = 199,360 gallons in a year.