TEACHER PAGES

# Oster Grove Paves a Field: Stormwater Runoff and Percent Change

Worksheet · Grade 7 · Science

A grade 7 science worksheet where students compare stormwater runoff from a grass field and a parking lot, using the runoff formula and percent change.

| Time | 45 minutes |
| --- | --- |
| Prep | No prep |
| Materials | None |
| Answer key | At the end of this document |
| License | Public domain (CC0). Copy, edit, and share freely, without attribution. |

## Objectives

- Students will calculate stormwater runoff volume from area, rainfall, and a runoff fraction.

- Students will compare runoff from a grass surface and a paved surface.

- Students will find the percent increase in runoff caused by paving a field.

## Standards

MS-ESS3-3 (NGSS): Apply scientific principles to design a method for monitoring and minimizing a human impact on the environment.

How this material addresses it: Items 2 through 7 have students quantify how paving a field increases runoff, the human-impact evidence this standard asks students to weigh when minimizing that impact.

7.RP.A.3 (Common Core Math): Use proportional relationships to solve multistep ratio and percent problems. Examples: simple interest, tax, markups and markdowns, gratuities and commissions, fees, percent increase and decrease, percent error.

How this material addresses it: Item 5 is a multistep percent problem finding the percent increase in runoff after paving, and item 7 applies the proportional runoff fraction to compare a half-paved plan against full pavement.

## Vocabulary

Stormwater: Stormwater is rain or melted snow that flows off roofs, streets, and parking lots. In most towns it goes into storm drains and flows to a stream without being treated, carrying whatever it picked up.

Also: runoff, impervious, percent increase

## Teacher overview

When rain falls on grass, most of it soaks into the ground; when it falls on pavement, most of it runs off into storm drains. This worksheet has students compare the two using one plot of land in a town that wants to pave a field for parking. The task builds percent-change practice on a real question that many towns face, and it lets students see how a land-use choice changes where the water goes.

## Differentiation

### Support

Provide the value from item 1 (24,920 gallons) already computed so students can begin at item 2, and give a worked model of finding a percent increase before item 5. Allow a calculator throughout and accept answers rounded to the nearest hundred gallons.

### Extension

Ask students to find how large a rain garden, in gallons of storage, the town would need to hold all the extra runoff from item 4 during one storm, and to explain any assumption they make.

### ELL supports

Pre-teach "runoff," "soak in," "pavement," and "percent increase" with a quick demonstration pouring water on a sponge and on a plastic lid. Provide a labeled diagram of rain hitting grass versus pavement, and allow numeric answers with short phrases.

Name	   Date

# Oster Grove Paves a Field: Stormwater Runoff and Percent Change

Grade 7 · Science

## Student Worksheet

The town of Oster Grove is deciding whether to pave a grassy field into a parking lot. Use these values and the runoff formula:

- The field is 20,000 square feet.

- A storm drops 2 inches of rain.

- Runoff formula: rain that falls (gallons) = area (sq ft) x rainfall (inches) x 0.623. Runoff = rain that falls x the fraction that runs off.

- As grass, about 20% (0.20) of the rain runs off. As pavement, about 90% (0.90) runs off.

- First find the total rain that falls on the field. Using the formula, how many gallons of rain land on the 20,000 square foot field during the 2-inch storm?

- As a grass field, only 0.20 of that rain runs off. How many gallons run off the grass field?

- As a parking lot, 0.90 of that rain runs off. How many gallons run off the parking lot?

- How many more gallons run off as pavement than as grass?

- Compared with the grass runoff in item 2, what is the percent increase in runoff after paving? Round to the nearest whole percent.

- As grass, the rain that does not run off soaks into the ground. How many gallons soak into the grass field?

- Oster Grove considers a compromise: keep half the field as grass (10,000 sq ft) and pave the other half (10,000 sq ft). Find the total runoff from this half-and-half plan, then state how many fewer gallons run off than with full pavement.

- Explain why more runoff from the parking lot could cause problems downstream, such as flooding or pollution reaching a creek.

- Name one design that would let a parking lot produce less runoff, and explain how it works.

ANSWER KEY · FOR TEACHERS

# Oster Grove Paves a Field: Stormwater Runoff and Percent Change

Worksheet · Grade 7 · Science

- Answer: 24,920 gallons. 20,000 sq ft x 2 in x 0.623 = 24,920 gallons of rain fall on the field.

- Answer: 4,984 gallons. 24,920 x 0.20 = 4,984 gallons run off the grass field.

- Answer: 22,428 gallons. 24,920 x 0.90 = 22,428 gallons run off the parking lot.

- Answer: 17,444 gallons. 22,428 - 4,984 = 17,444 more gallons run off as pavement.

- Answer: 350 percent. The increase is 17,444 gallons; 17,444 / 4,984 = 3.5, which is a 350% increase.

- Answer: 19,936 gallons. The rain that soaks in is 0.80 of the total; 24,920 x 0.80 = 19,936 gallons.

- Answer: 13,706 gallons, which is 8,722 fewer than full pavement. Each half receives 10,000 x 2 x 0.623 = 12,460 gallons; grass half runoff is 12,460 x 0.20 = 2,492 and pavement half is 12,460 x 0.90 = 11,214, totaling 13,706 gallons. Full pavement runs off 22,428, so the plan saves 22,428 - 13,706 = 8,722 gallons.

- Answer: Pavement sends far more water to storm drains at once, which can overflow drains and flood streets, and the fast-moving water carries oil, litter, and other pollutants from the lot into nearby creeks.

- Answer: Accept any reasonable answer, such as permeable pavement that lets water soak through, a rain garden that collects runoff, or a detention basin that holds water and releases it slowly, with a short explanation of how it reduces runoff.